AT-STLN-ARTIX 7-001

FPGA Development Board

Hardware Reference Manual

Anmaya Technologies

Product Code: ANM-PRD-2025-005  |  Rev: 1.0

Schematic Tool: KiCad E.D.A. 9.0.4

# 1. Board Overview

The AT-STLN-ARTIX 7-001 is a feature-rich FPGA development board designed by Anmaya Technologies for educational and prototyping purposes. It is built around the Xilinx XC7A35T-FTG256-1 (Artix-7 series) FPGA and integrates a wide range of peripherals, actuators, and communication interfaces on a single PCB.

## 1.1 Key Features

- FPGA: Xilinx XC7A35T-FTG256-1 (Artix-7, 33K logic cells, 256-ball FTBGA, Speed Grade -1)

- USB Interface: FTDI FT232H high-speed USB-to-JTAG/UART bridge

- Configuration: MX25L3233FM1 32Mb SPI Flash for bitstream storage

- Memory: IS61WV25616EDBLL 512K x 16-bit asynchronous SRAM

- DAC: AD9744 14-bit, 210 MSPS high-speed DAC with OPA356AID output op-amp

- ADC: MCP3202 12-bit, 2-channel SPI ADC

- Display: 16x2 character LCD (DS1WC1602A) + 4-digit 7-segment display (MAX7219 driver)

- Wireless: ESP32-C3-MINI-1 Wi-Fi/BLE co-processor

- Motors: DC motor driver (DRV8833) + stepper motor driver (L293DD)

- I/O: PMOD connector (20-pin, 16 user IOs), 4x4 matrix keypad, 16 user LEDs, 8 slide switches, 6 push buttons

- Relay: Single SANYOU SRD Form-C relay (5V coil, 10A contacts)

- Buzzer: Active piezo buzzer (BC817 NPN driver)

- Storage: Micro SD card slot

- Power: Multi-rail power supply (5V, 3.3V, 1.8V, 1.0V) from 15V DC barrel jack

- Clock: 24 MHz system oscillator (ASDM1) + 12 MHz USB crystal

# 2. FPGA Specifications

## 2.1 XC7A35T-FTG256-1 Key Parameters

| Parameter | Value |
| --- | --- |
| Device Family | Xilinx Artix-7 |
| Part Number | XC7A35T-FTG256-1 |
| Speed Grade | -1 (slowest / most power-efficient commercial grade) |
| Package | FTG256 (FTBGA, 256-ball, 17×17 mm) |
| Logic Cells | 33,280 |
| CLB Flip-Flops | 41,600 (20,800 LUTs) |
| CLB Slices | 8,150 |
| Distributed RAM | 400 Kb |
| Block RAM (36Kb tiles) | 50 tiles (1,800 Kb total) |
| DSP48E1 Slices | 90 |
| XADC (12-bit ADC) | 1 block (dual 12-bit, 1 MSPS, up to 17 channels) |
| Max User I/O | 170 |
| PLLs / MMCMs | 2 PLLs, 2 MMCMs |
| VCCINT (Core) | 1.0 V |
| VCCAUX | 1.8 V |
| VCCO (I/O Banks) | 3.3 V (banks 14, 15, 34, 35) |
| Configuration | Master SPI (M[2:0]=001) / JTAG (M[2:0]=101) |
| SPI Flash | MX25L3233FM1 (32Mb, 3.3V) |
| Temperature Range | 0°C to +85°C (Commercial grade) |

# 3. FPGA Pin Mapping

The following tables list the complete FPGA pin assignments derived from the schematic (ANM-PRD-2025-005). The FPGA has four active I/O banks used for user logic: Banks 14, 15, 34, and 35.

## 3.1 User LEDs (Generic)

| Net Name | FPGA Pin (Bank 35) |
| --- | --- |
| LED1 | D5 |
| LED2 | A3 |
| LED3 | B4 |
| LED4 | A4 |
| LED5 | E6 |
| LED6 | C13 |
| LED7 | C14 |
| LED8 | D14 |

## 3.2 7-Segment Display (MAX7219 — SPI Interface)

The 4-digit 7-segment display (U15, XLITX 3641AS) is driven by a MAX7219 (U14) via SPI. The FPGA drives 3.3V signals through SN74LVC1T45 level-shifters (U17/U18/U19) to the 5V MAX7219.

| Signal | FPGA Net | FPGA Pin (Bank 15) | Direction | Description |
| --- | --- | --- | --- | --- |
| 7SEG_{DIN} | 7SEG_DIN | J15 | FPGA → MAX7219 | Serial data in |
| 7SEG_{LOAD} | 7SEG_LOAD | J16 | FPGA → MAX7219 | Chip select / load |
| 7SEG_{CLK} | 7SEG_CLK | H12 | FPGA → MAX7219 | Serial clock |

## 4.3 16x2 LCD Interface

The LCD (DS1WC1602A) operates on 5V. It is connected in 8-bit mode to FPGA Bank 35 I/Os. Contrast is set by potentiometer RV1.

| LCD Pin | Net Name | FPGA Pin (Bank 35) | Direction |
| --- | --- | --- | --- |
| RS (Pin 4) | LCD_RS | G4 | FPGA → LCD |
| R/W (Pin 5) | LCD_R/W | H3 | FPGA → LCD |
| E (Pin 6) | LCD_E | E1 | FPGA → LCD |
| D0 (Pin 7) | LCD_D0 | G2 | FPGA ↔ LCD |
| D1 (Pin 8) | LCD_D1 | G1 | FPGA ↔ LCD |
| D2 (Pin 9) | LCD_D2 | H5 | FPGA ↔ LCD |
| D3 (Pin 10) | LCD_D3 | H4 | FPGA ↔ LCD |
| D4 (Pin 11) | LCD_D4 | J5 | FPGA ↔ LCD |
| D5 (Pin 12) | LCD_D5 | J4 | FPGA ↔ LCD |
| D6 (Pin 13) | LCD_D6 | H2 | FPGA ↔ LCD |
| D7 (Pin 14) | LCD_D7 | H1 | FPGA ↔ LCD |

## 3.4 Slide Switches (SW0–SW15)

16 FPGA-connected slide switches (SPDT) with 220Ω series resistors and pull-up/pull-down to 3.3V. Mapped to FPGA Bank 35.

| Switch Net | FPGA Pin |
| --- | --- |
| SS0 | C9 |
| SS1 | B9 |
| SS2 | G5 |
| SS3 | A7 |
| SS4 | C7 |
| SS5 | A10 |
| SS6 | B7 |
| SS7 | A8 |

## 3.5 4×4 Matrix Keypad

The 4x4 keypad (keys 0-9, A-F) connects to FPGA Bank 35 via 10K pull-up resistors and 100nF debounce caps. Keys are arranged as rows (SW1–SW4) × columns (SW5–SW8 pull groups).

| Keypad Net | FPGA Pin | Keypad Net | FPGA Pin |
| --- | --- | --- | --- |
| 0 | A13 | 8 | A5 |
| 1 | F5 | 9 | C6 |
| 2 | E3 | A | D4 |
| 3 | F2 | B | F4 |
| 4 | A12 | C | B6 |
| 5 | D6 | D | B5 |
| 6 | D3 | E | C4 |
| 7 | F3 | F | E5 |

## 3.7 PMOD Connector (J16 — 20 Pin)

The PMOD connector (J16) exposes 16 FPGA I/O signals from Bank 34 with 0Ω series resistors (R112–R127) for easy daughterboard attachment. GND on pin 19, 3.3V on pin 20.

| J16 Pin | FPGA Pin | J16 Pin | FPGA Pin |
| --- | --- | --- | --- |
| IO_0 | T2 | IO_8 | N1 |
| IO_1 | R3 | IO_9 | N2 |
| IO_2 | T3 | IO_10 | R2 |
| IO_3 | T4 | IO_11 | N3 |
| IO_4 | M1 | IO_12 | P3 |
| IO_5 | M2 | IO_13 | N4 |
| IO_6 | P1 | IO_14 | P4 |
| IO_7 | _R1 | IO_15 | _P5 |

## 3.8 SPI Bus (Shared — SPIm)

A shared SPI master bus connects the FPGA to the ADC (MCP3202), SD card, and ESP32. Chip selects are individual.

| Signal | FPGA Pin (Bank 15) | Connected To |
| --- | --- | --- |
| SPIm_SCK | G11 (IO_25_15) | ESP GPIO5/ADC2_CH0 |
| SPIm_MOSI | G12 (IO_L19N_T3_A21_VREF_15) | MCP3202 DIN, SD CMD, ESP GPIO7 |
| SPIm_MISO | G14 (IO_L19N_T3_A21_VREF_15) | MCP3202 DOUT, SD DAT0, ESP GPIO4 |
| SD_CS | H14 (IO_L24P_T3_RS1_15) | SD card DAT3/CD |
| ESP_CS | F14 (IO_L21N_T3_DQS_A18_15) | ESP32 GPIO6 |
| ADC uses SD_CS net | See ADC section | MCP3202 ~CS~ |

## 3.9 Clock Inputs

| Clock Signal | FPGA Pin | Bank | Source | Frequency |
| --- | --- | --- | --- | --- |
| GCLK_24MHz | D13 (IO_L12P_T1_MRCC_15) | 15 | ASDM1-24MHz oscillator (U4) | 24 MHz |
| EMCCLK | M15 (IO_L3N_T0_DQS_EMCCLK_14) | 14 | FT232H ACBUS pin | Ext. clock |

## 3.10 FPGA Configuration Pins

| Signal | FPGA Ball | Description |
| --- | --- | --- |
| CCLK_0 | E8 | Configuration clock (SPI) |
| M0_0 | M9 | Mode select 0 — SW17 (default: 1 = SPI master) |
| M1_0 | M10 | Mode select 1 — SW36 (default: 0) |
| M2_0 | M11 | Mode select 2 — SW37 (default: 0) |
| DONE_0 | H10 | Configuration done indicator (LED D2) |
| INIT_B_0 | K10 | Initialization busy (LED D1, active low) |
| PROGRAM_B_0 | L9 | Reprogram trigger (BTN2 PROGRAM, active low) |
| CFGBVS_0 | E7 | Config bank voltage select — pulled to 3.3V |
| TDI_0 | N7 | JTAG Test Data In |
| TDO_0 | N8 | JTAG Test Data Out |
| TMS_0 | M7 | JTAG Test Mode Select |
| TCK_0 | L7 | JTAG Test Clock |

# 4. Peripheral Interfaces

## 4.1 USB JTAG / UART (FT232H — U2)

The FT232H (U2) provides USB 2.0 HS to JTAG and UART conversion via a USB-C connector (J1). It is configured using a 93LC56B EEPROM (U10). JTAG signals pass through 100Ω series resistors and ESD protection diodes.

| FT232H Signal | Net | FPGA Ball | Notes |
| --- | --- | --- | --- |
| ADBUS0 | JTAG_R_TDI → JTAG_TDI | N7 | TCK via 100Ω R130 |
| ADBUS1 | JTAG_R_TDO → JTAG_TDO | N8 | 100Ω R131 |
| ADBUS2 | JTAG_R_TMS → JTAG_TMS | M7 | 100Ω R132 |
| ADBUS3 | JTAG_R_TCK → JTAG_TCK | L7 | 100Ω R133 |
| ACBUS0-9 | GPIO | — | Unused / test points |
| DM/DP | USB_D- / USB_D+ | J1 connector | USB 2.0 data pair |

## 4.2 JTAG Header (J3 — 14-Pin)

| J3 Pin | Signal | J3 Pin | Signal |
| --- | --- | --- | --- |
| 1 | JTAG_TMS | 2 | +3V3 |
| 3 | GND | 4 | JTAG_TCK |
| 5 | GND | 6 | JTAG_TDO |
| 7 | GND | 8 | JTAG_TDI |
| 9 | GND | 10 | — |
| 11 | GND | 12 | — |
| 13 | GND | 14 | — |

## 4.3 DAC (AD9744 — U24)

The AD9744 is a 14-bit, 210 MSPS current-output DAC. Its digital inputs connect via 22Ω damping resistors to FPGA Bank 14/15. The differential current outputs (IOUTA/IOUTB) feed an OPA356AID op-amp for single-ended voltage output on J5.

| DAC Signal | FPGA Net | FPGA Pin | DAC Signal | FPGA Net | FPGA Pin |
| --- | --- | --- | --- | --- | --- |
| DAC_D0 | R_DAC_D0 | H16 | DAC_D7 | R_DAC_D7 | D16 |
| DAC_D1 | R_DAC_D1 | G15 | DAC_D8 | R_DAC_D8 | B14 |
| DAC_D2 | R_DAC_D2 | G16 | DAC_D9 | R_DAC_D9 | A15 |
| DAC_D3 | R_DAC_D3 | F13 | DAC_D10 | R_DAC_D10 | B15 |
| DAC_D4 | R_DAC_D4 | F15 | DAC_D11 | R_DAC_D11 | D15 |
| DAC_D5 | R_DAC_D5 | E16 | DAC_D12 | R_DAC_D12 | C16 |
| DAC_D6 | R_DAC_D6 | E15 | DAC_D13 | R_DAC_D13 | B16 |
| DAC_CLK | R_DAC_CLK | A14 | DAC_MODE | R_DAC_MODE | H13 |

## 4.4 ADC (MCP3202 — U26)

The MCP3202 is a 12-bit, 2-channel SPI ADC. CH0 accepts analog from the on-board LM35 temperature sensor or screw terminal (J8). CH1 connects to potentiometer RV2 for manual analog input.

| Signal | FPGA Pin | Description |
| --- | --- | --- |
| SPIm_SCK | G11 | ADC clock (shared SPI bus) |
| SPIm_MOSI | G12 | Data to ADC (DIN) |
| SPIm_MISO | G14 | Data from ADC (DOUT) |
| SD_CS (10K pull-up) | H14 | ADC chip select (active low) |
| ADC_CH0 | Screw J8 pin 1 | External analog input or LM35 output |
| ADC_CH1 | RV2 wiper | On-board pot (0–3.3V range) |

## 4.5 SRAM (IS61WV25616EDBLL — U3)

512K × 16-bit asynchronous SRAM, 3.3V, operating at up to 10 ns access time. All 18 address bits and 16 data bits are connected to FPGA Bank 14.

| Signal Group | Nets | FPGA Bank | Bit Width |
| --- | --- | --- | --- |
| Address | SRAM_A0 – SRAM_A17 | Bank 14 | 18 bits |
| Data | SRAM_D0 – SRAM_D15 | Bank 14 | 16 bits |
| Chip Enable | SRAM_CE | Bank 14 | 1 bit (active low) |
| Output Enable | SRAM_OE | Bank 14 | 1 bit (active low) |
| Write Enable | SRAM_WE | Bank 14 | 1 bit (active low) |
| Upper Byte | SRAM_UB | Bank 14 | 1 bit (active low) |
| Lower Byte | SRAM_LB | Bank 14 | 1 bit (active low) |

| Nets | FPGA Pin | Nets | FPGA Pin | Nets | FPGA Pin |
| --- | --- | --- | --- | --- | --- |
| SRAM_A0 | T12 | SRAM_A13 | T9 | SRAM_D8 | T10 |
| SRAM_A1 | R12 | SRAM_A14 | T5 | SRAM_D9 | N9 |
| SRAM_A2 | T13 | SRAM_A15 | R6 | SRAM_D10 | P9 |
| SRAM_A3 | T14 | SRAM_A16 | R6 | SRAM_D11 | N6 |
| SRAM_A4 | T15 | SRAM_A17 | R5 | SRAM_D12 | R10 |
| SRAM_A5 | P13 | SRAM_D0 | R16 | SRAM_D13 | P10 |
| SRAM_A6 | R13 | SRAM_D1 | P15 | SRAM_D14 | M12 |
| SRAM_A7 | N12 | SRAM_D2 | P16 | SRAM_D15 | R11 |
| SRAM_A8 | N11 | SRAM_D3 | N16 | SRAM_CE | R15 |
| SRAM_A9 | P11 | SRAM_D4 | M16 | SRAM_OE | P6 |
| SRAM_A10 | R7 | SRAM_D5 | M14 | SRAM_WE | N13 |
| SRAM_A11 | T8 | SRAM_D6 | N14 | SRAM_UB | P8 |
| SRAM_A12 | R8 | SRAM_D7 | P14 | SRAM_LB | T7 |

## 4.6 Micro SD Card (J6)

| SD Signal | Net | FPGA Pin | Description |
| --- | --- | --- | --- |
| DAT0 | SD_D0 | D8 (Bank 15) | MISO / Data bit 0 |
| DAT1 | SD_D1 | D9 (Bank 15) | Data bit 1 |
| DAT2 | SD_D2 | B10(Bank 15) | Data bit 2 |
| DAT3/CD | SD_D3 | B11 (Bank 15) | CS / Card Detect dual-use |
| CMD | SD_CMD | B12 (Bank 15) | MOSI / Command |
| CLK | SD_SCK | C11 (Bank 15) | Clock |
| DET | SD_DET | C12 (Bank 15) | Card detect switch |

## 4.7 ESP32-C3-MINI-1 (U13)

The ESP32-C3-MINI-1 serves as a Wi-Fi/BLE co-processor. It connects to the FPGA via shared SPI and UART, and has its own USB-C port (J9) for programming.

| ESP32 GPIO | Net / Function | FPGA Pin / Connection |
| --- | --- | --- |
| GPIO4 | SPIm_MISO | FPGA G14 (Bank 15) |
| GPIO5 | SPIm_SCK | FPGA G11 (Bank 15) |
| GPIO6 | ESP_CS | FPGA F14 (Bank 15) |
| GPIO7 | SPIm_MOSI | FPGA G12 (Bank 15) |
| GPIO20 (U0RXD) | ESP_RX | J13 connector pin 5 |
| GPIO21 (U0TXD) | ESP_TX | J13 connector pin 6 |
| GPIO18 (USB_D-) | ESP_N | J9 USB-C D− (0Ω R20) |
| GPIO19 (USB_D+) | ESP_P | J9 USB-C D+ (0Ω R21) |
| GPIO0/1/3/10 | ESP_IO0/1/3/10 | J13 connector pins 2/3/1/4 |
| EN/CHIP_PU | NRST | RC filter (C90/C91) + SW19 for BOOT |

## 4.8 DC Motor Driver (DRV8833 — U28)

| Signal | FPGA Net | FPGA Pin | Description |
| --- | --- | --- | --- |
| IN1 | IN1 | F12 (Bank 14) | Motor A direction input 1 |
| IN2 | IN2 | H11(Bank 14) | Motor A direction input 2 |

Current sensing via 200mΩ resistors (R193, R195) at TP10/TP11. Motor M1 connects to AOUT1/AOUT2. Sleep controlled by FPGA (or pulled high for always-on).

## 4.9 Stepper Motor Driver (L293DD — U16)

| Signal | FPGA Net | FPGA Pin (Bank 15) | L293DD Pin |
| --- | --- | --- | --- |
| SIG1 | SIG1 | E12 | INPUT_1 (pin 2) |
| SIG2 | SIG2 | E13 | INPUT_2 (pin 9) |
| SIG3 | SIG3 | E11 | INPUT_3 (pin 12) |
| SIG4 | SIG4 | D11 | INPUT_4 (pin 19) |

Outputs SM1–SM4 go to J7 screw terminal (5-pin: SM1, SM2, SM3, SM4, +5V). Enable pins are tied high (+5V).

## 4.10 Relay (K1 — SANYOU SRD Form-C)

| Signal | FPGA Net | FPGA Pin | Notes |
| --- | --- | --- | --- |
| RLY_IN | RLY_IN | L5 (IO_0_34, Bank 34) | High = relay energized via Q7 (SS8050 NPN) |

Relay coil powered from +5V via ferrite FB9. J17 (3-pin screw): NO, COM, NC. Max contacts: 10A @ 125VAC / 10A @ 30VDC.

## 4.11 Buzzer (BZ1)

| Signal | FPGA Net | FPGA Pin | Notes |
| --- | --- | --- | --- |
| BZR_OUT | BZR_OUT | K5 (IO_25_35, Bank 35) | Drives BC817 NPN transistor; 100Ω base resistor |

# 5. Configuration & Programming

## 5.1 Configuration Modes

The configuration mode is selected by three SPDT slide switches on the board:

| Mode | SW17 (M2) | SW36 (M1) | SW37 (M0) | Description |
| --- | --- | --- | --- | --- |
| Master SPI | 0 | 0 | 1 | Boot from SPI Flash (MX25L3233FM1) — default |
| JTAG | 1 | 0 | 1 | Load bitstream directly via USB JTAG |

## 5.2 Programming via Vivado

Connect USB-C cable to J1. Use Vivado Hardware Manager to detect the xc7a50t_0 target. For remote lab use, the FT232H enumerates as a JTAG adapter compatible with Xilinx cable drivers.

- Bitstream file: .bit (generated by Vivado implementation)

- Flash programming: Use Vivado's 'Add Configuration Memory Device' → MX25L3233F

- SPI flash address: 0x00000000 (default)

# 6. XDC Constraints Reference

Below is a complete XDC template covering all major peripherals. Adapt as needed for your Verilog top-level port names.

## Clock

create_clock -period 41.667 -name sys_clk [get_ports clk_24mhz]

set_property -dict {PACKAGE_PIN D13 IOSTANDARD LVCMOS33} [get_ports clk_24mhz]

## User LEDs (L1-L8 shown; extend to L16)

set_property -dict {PACKAGE_PIN D5  IOSTANDARD LVCMOS33} [get_ports led[0]]

set_property -dict {PACKAGE_PIN A3  IOSTANDARD LVCMOS33} [get_ports led[1]]

set_property -dict {PACKAGE_PIN B4  IOSTANDARD LVCMOS33} [get_ports led[2]]

set_property -dict {PACKAGE_PIN A4  IOSTANDARD LVCMOS33} [get_ports led[3]]

set_property -dict {PACKAGE_PIN E6  IOSTANDARD LVCMOS33} [get_ports led[4]]

set_property -dict {PACKAGE_PIN C13  IOSTANDARD LVCMOS33} [get_ports led[5]]

set_property -dict {PACKAGE_PIN C14 IOSTANDARD LVCMOS33} [get_ports led[6]]

set_property -dict {PACKAGE_PIN D14 IOSTANDARD LVCMOS33} [get_ports led[7]]

## Slide Switches

set_property -dict {PACKAGE_PIN C9  IOSTANDARD LVCMOS33} [get_ports sw[0]]

set_property -dict {PACKAGE_PIN B9  IOSTANDARD LVCMOS33} [get_ports sw[1]]

set_property -dict {PACKAGE_PIN G5  IOSTANDARD LVCMOS33} [get_ports sw[2]]

set_property -dict {PACKAGE_PIN A7  IOSTANDARD LVCMOS33} [get_ports sw[3]]

set_property -dict {PACKAGE_PIN C7 IOSTANDARD LVCMOS33} [get_ports sw[4]]

set_property -dict {PACKAGE_PIN A10 IOSTANDARD LVCMOS33} [get_ports sw[5]]

set_property -dict {PACKAGE_PIN B7 IOSTANDARD LVCMOS33} [get_ports sw[6]]

set_property -dict {PACKAGE_PIN A8 IOSTANDARD LVCMOS33} [get_ports sw[7]]

## 7-Segment Display (MAX7219 SPI)

set_property -dict {PACKAGE_PIN J15 IOSTANDARD LVCMOS33} [get_ports seg_din]

set_property -dict {PACKAGE_PIN J16 IOSTANDARD LVCMOS33} [get_ports seg_load]

set_property -dict {PACKAGE_PIN H12 IOSTANDARD LVCMOS33} [get_ports seg_clk]

## LCD (16x2)

set_property -dict {PACKAGE_PIN G4  IOSTANDARD LVCMOS33} [get_ports lcd_rs]

set_property -dict {PACKAGE_PIN H3  IOSTANDARD LVCMOS33} [get_ports lcd_rw]

set_property -dict {PACKAGE_PIN E1  IOSTANDARD LVCMOS33} [get_ports lcd_en]

set_property -dict {PACKAGE_PIN G2  IOSTANDARD LVCMOS33} [get_ports lcd_d[0]]

set_property -dict {PACKAGE_PIN G1  IOSTANDARD LVCMOS33} [get_ports lcd_d[1]]

set_property -dict {PACKAGE_PIN H5  IOSTANDARD LVCMOS33} [get_ports lcd_d[2]]

set_property -dict {PACKAGE_PIN H4  IOSTANDARD LVCMOS33} [get_ports lcd_d[3]]

set_property -dict {PACKAGE_PIN J5  IOSTANDARD LVCMOS33} [get_ports lcd_d[4]]

set_property -dict {PACKAGE_PIN J4  IOSTANDARD LVCMOS33} [get_ports lcd_d[5]]

set_property -dict {PACKAGE_PIN H2  IOSTANDARD LVCMOS33} [get_ports lcd_d[6]]

set_property -dict {PACKAGE_PIN H1  IOSTANDARD LVCMOS33} [get_ports lcd_d[7]]

## Buzzer

set_property -dict {PACKAGE_PIN K5  IOSTANDARD LVCMOS33} [get_ports buzzer]

## Relay

set_property -dict {PACKAGE_PIN L5  IOSTANDARD LVCMOS33} [get_ports relay_in]

## SPI (Shared Bus)

set_property -dict {PACKAGE_PIN G11 IOSTANDARD LVCMOS33} [get_ports spi_sck]

set_property -dict {PACKAGE_PIN G14 IOSTANDARD LVCMOS33} [get_ports spi_mosi]

set_property -dict {PACKAGE_PIN G12 IOSTANDARD LVCMOS33} [get_ports spi_miso]

set_property -dict {PACKAGE_PIN H14 IOSTANDARD LVCMOS33} [get_ports sd_cs]

## PMOD (IO_0–IO_7 shown)

set_property -dict {PACKAGE_PIN T2  IOSTANDARD LVCMOS33} [get_ports pmod[0]]

set_property -dict {PACKAGE_PIN R3  IOSTANDARD LVCMOS33} [get_ports pmod[1]]

set_property -dict {PACKAGE_PIN T3  IOSTANDARD LVCMOS33} [get_ports pmod[2]]

set_property -dict {PACKAGE_PIN T4  IOSTANDARD LVCMOS33} [get_ports pmod[3]]

set_property -dict {PACKAGE_PIN M1  IOSTANDARD LVCMOS33} [get_ports pmod[4]]

set_property -dict {PACKAGE_PIN M2  IOSTANDARD LVCMOS33} [get_ports pmod[5]]

set_property -dict {PACKAGE_PIN P1  IOSTANDARD LVCMOS33} [get_ports pmod[6]]

set_property -dict {PACKAGE_PIN R1  IOSTANDARD LVCMOS33} [get_ports pmod[7]]

# 7. Quick-Start Guide

## 7.1 Hardware Setup

- Connect 15V DC power supply to J2 (barrel jack). Verify the PWR 5V0, PWR 3V3, PWR 1V8, and PWR 1V0 LEDs all illuminate.

- Set configuration mode switches: SW17 (M2) = OFF, SW36 (M1) = OFF, SW37 (M0) = ON for Master SPI mode.

- Connect USB-C cable to J1 (FT232H). Install Xilinx FTDI drivers if not present.

- Open Vivado → Hardware Manager → Open Target → Auto Connect. The xc7a50t_0 device should appear.

- Program Device → select your .bit file → Program.

- DONE LED (D2) should illuminate after successful configuration.

## 7.2 Minimal Blink XDC + Verilog

Use clock pin D13 (24 MHz), LEDs B7–B5, switches A7–A8. A 24-bit prescaler gives ~1.4 Hz blink at 24 MHz.

## 7.3 Re-programming the Board

- Press BTN2 (PROGRAM) to pulse PROGRAM_B low — this triggers reconfiguration from SPI flash.

- For a soft reset, toggle PROGRAM_B via Vivado TCL: set_property PROGRAM.RESET_AFTER_PROGRAM true.